# TCPDump Automated Test Report

Engineer: divya

Date: 31.12.2025

TCPDump Version: 4.99.5

# Test Summary

| Test ID | Description | Status | Expected String |
| --- | --- | --- | --- |
| TC1 | Check tcpdump version | PASS | tcpdump |
| TC2 | Capture 10 packets | PASS | CAPTURE_OK |
| TC3 | Capture only ICMP packets | PASS | ICMP_OK |
| TC4 | Filter packets with IP 8.8.8.8 | PASS | FILTER_OK |
| TC5 | Packet monitoring | PASS | BANDWIDTH_OK |

# Detailed Test Results

## TC1

Description: Check tcpdump version

Pre-Actions: None

Check Command:
tcpdump --version | head -n 1

Expected Output Substring: tcpdump

Actual Output:

tcpdump version 4.99.5

Error Output:

NO ERROR

Final Status: PASS

Notes: None

----------------------------------------

## TC2

Description: Capture 10 packets

Pre-Actions:

ping -c 20 google.com

Check Command:
sudo timeout {timeout} tcpdump -i {iface} -c 10 -nn -tt ; echo CAPTURE_OK

Expected Output Substring: CAPTURE_OK

Actual Output:

1767180391.246202 IP 10.80.49.238.62122 > 10.80.49.142.22: Flags [.], ack 3969959000, win 250, length 0
1767180392.205243 IP 10.80.49.142.22 > 10.80.49.238.62122: Flags [P.], seq 1:69, ack 0, win 501, length 68
1767180392.212741 ARP, Request who-has 10.80.49.244 tell 10.80.49.142, length 28
1767180392.213359 IP6 fe80::20c:29ff:fe68:d61b > ff02::1:ff00:ac: ICMP6, neighbor solicitation, who has 2401:4900:ca80:8431::ac, length 32
1767180392.219814 ARP, Reply 10.80.49.244 is-at 1a:cc:90:b4:ed:7c, length 46
1767180392.219844 IP 10.80.49.142.51492 > 10.80.49.244.53: 22910+ [1au] A? google.com. (39)
1767180392.220049 IP 10.80.49.142.59237 > 10.80.49.244.53: 13897+ [1au] AAAA? google.com. (39)
1767180392.221150 IP6 2401:4900:ca80:8431::ac > fe80::20c:29ff:fe68:d61b: ICMP6, neighbor advertisement, tgt is 2401:4900:ca80:8431::ac, length 32
1767180392.221176 IP6 2401:4900:ca80:8431:20c:29ff:fe68:d625.44735 > 2401:4900:ca80:8431::ac.53: 52179+ [1au] A? google.com. (39)
1767180392.221233 IP6 2401:4900:ca80:8431:20c:29ff:fe68:d625.35968 > 2401:4900:ca80:8431::ac.53: 3352+ [1au] AAAA? google.com. (39)
CAPTURE_OK

Error Output:

tcpdump: verbose output suppressed, use -v[v]... for full protocol decode
listening on ens33, link-type EN10MB (Ethernet), snapshot length 262144 bytes
10 packets captured
10 packets received by filter
0 packets dropped by kernel

Final Status: PASS

Notes: None

----------------------------------------

## TC3

Description: Capture only ICMP packets

Pre-Actions:

ping -c 5 -I {iface} 8.8.8.8

Check Command:
sudo timeout {timeout} tcpdump -i {iface} icmp -c 5 -nn -tt ; echo ICMP_OK

Expected Output Substring: ICMP_OK

Actual Output:

1767180412.469042 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 6, seq 1, length 64
1767180412.490556 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 6, seq 1, length 64
1767180413.470965 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 6, seq 2, length 64
1767180413.494590 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 6, seq 2, length 64
1767180414.472893 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 6, seq 3, length 64
ICMP_OK

Error Output:

tcpdump: verbose output suppressed, use -v[v]... for full protocol decode
listening on ens33, link-type EN10MB (Ethernet), snapshot length 262144 bytes
5 packets captured
6 packets received by filter
0 packets dropped by kernel

Final Status: PASS

Notes: None

----------------------------------------

## TC4

Description: Filter packets with IP 8.8.8.8

Pre-Actions:

ping -c 5 -I {iface} 8.8.8.8

Check Command:
sudo timeout {timeout} tcpdump -i {iface} host 8.8.8.8 -c 3 -nn -tt ; echo FILTER_OK

Expected Output Substring: FILTER_OK

Actual Output:

1767180417.508204 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 7, seq 1, length 64
1767180417.547833 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 7, seq 1, length 64
1767180418.510249 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 7, seq 2, length 64
FILTER_OK

Error Output:

tcpdump: verbose output suppressed, use -v[v]... for full protocol decode
listening on ens33, link-type EN10MB (Ethernet), snapshot length 262144 bytes
3 packets captured
4 packets received by filter
0 packets dropped by kernel

Final Status: PASS

Notes: None

----------------------------------------

## TC5

Description: Packet monitoring

Pre-Actions:

ping -c 30 -I {iface} 8.8.8.8

Check Command:
sudo timeout {timeout} tcpdump -i {iface} -c 30 -nn -tt | awk '{
       print $0;
       for(i=1;i<=NF;i++)
         if($i=="length")
           sum+=$(i+1)
     }
     END{
       bw=(sum*8)/{timeout};
       printf "\nTotal Bytes: %d\nBandwidth: %.2f bps\n", sum, bw
     }';
echo BANDWIDTH_OK

Expected Output Substring: BANDWIDTH_OK

Actual Output:

1767180422.551584 IP 10.80.49.142.22 > 10.80.49.238.62122: Flags [P.], seq 3969966488:3969966564, ack 3840722265, win 501, length 76
1767180422.558285 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 1, length 64
1767180422.576655 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 1, length 64
1767180422.604133 IP 10.80.49.238.62122 > 10.80.49.142.22: Flags [.], ack 76, win 255, length 0
1767180423.559992 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 2, length 64
1767180423.585987 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 2, length 64
1767180423.636421 ARP, Request who-has 10.80.49.244 tell 10.80.49.142, length 28
1767180423.644433 ARP, Reply 10.80.49.244 is-at 1a:cc:90:b4:ed:7c, length 46
1767180423.675283 ARP, Request who-has 10.80.49.142 tell 10.80.49.244, length 46
1767180423.675319 ARP, Reply 10.80.49.142 is-at 00:0c:29:68:d6:1b, length 28
1767180424.561285 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 3, length 64
1767180424.586789 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 3, length 64
1767180424.901661 IP 0.0.0.0.68 > 255.255.255.255.67: BOOTP/DHCP, Request from 00:0c:29:65:16:96, length 299
1767180425.563089 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 4, length 64
1767180425.584312 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 4, length 64
1767180426.564662 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 5, length 64
1767180426.586233 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 5, length 64
1767180427.566543 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 6, length 64
1767180427.597724 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 6, length 64
1767180428.568016 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 7, length 64
1767180428.588317 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 7, length 64
1767180429.569662 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 8, length 64
1767180429.591258 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 8, length 64
1767180430.571574 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 9, length 64
1767180430.597335 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 9, length 64
1767180431.573666 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 10, length 64
1767180431.598549 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 10, length 64
1767180432.575231 IP 10.80.49.142 > 8.8.8.8: ICMP echo request, id 8, seq 11, length 64
1767180432.598605 IP 8.8.8.8 > 10.80.49.142: ICMP echo reply, id 8, seq 11, length 64


Total Bytes: 1931
Bandwidth: 1287.33 bps
BANDWIDTH_OK

Error Output:

tcpdump: verbose output suppressed, use -v[v]... for full protocol decode
listening on ens33, link-type EN10MB (Ethernet), snapshot length 262144 bytes
29 packets captured
29 packets received by filter
0 packets dropped by kernel

Final Status: PASS

Notes: None

----------------------------------------